DA2 | FULL IMPLEMENTATION

Name & Reg No: Mohnish 23BCE1081, Adavan 23BCE1053

GAZETYPE PROJECT WORKING & DETAILS

Keyboard Typing with Eyes

# 1. Project Overview

Keyboard Typing with Eyes is an assistive eye-controlled virtual keyboard. It allows a user to select characters without a physical keyboard by looking toward a left or right keyboard region and using blinks as deliberate control actions. A webcam captures the user, computer-vision landmarks estimate eye position and eye openness, and the selected input is displayed on an on-screen text board.

The current implementation is a real-time desktop prototype. It focuses on a complete interaction cycle: startup calibration, gaze-based keyboard selection, automatic key highlighting, blink-based confirmation, text editing, audio feedback, and safe exit.

# 2. Project Working

The system works as a sequence of vision-processing and user-interface stages. Each stage produces the information required by the next stage.

Table 1. End-to-end working sequence

| Stage | Detailed working | System output |
| --- | --- | --- |
| 1. Camera input | OpenCV captures frames from the configured webcam. Each frame is flipped horizontally for a natural mirror-style interaction. | Live camera frame |
| 2. Face processing | The frame is converted from BGR to RGB and passed to MediaPipe Face Landmarker in video mode. | Tracked facial landmarks |
| 3. Eye calibration | The system collects open-eye measurements and then records three normal blinks to establish a user-specific blink threshold. | Personalized blink detection |
| 4. Gaze selection | The iris position is normalized against the eye corners. Sustained gaze below or above the configured thresholds selects the left or right keyboard. | Selected keyboard side |
| 5. Key cycling | The selected keyboard cycles through its 15 positions at the configured frame interval while the user keeps their eyes open. | Highlighted key |
| 6. Blink control | The first blink pauses the highlighted key. A second blink within the confirmation timeout selects it. | Confirmed key action |
| 7. Text output | Letters are appended, SPACE inserts a blank, and BACK removes the last character. The board and character count are refreshed. | Visible typed text |

# 3. Packages and Technologies Used

The project combines Python libraries for computer vision, numerical processing, face landmark inference, and audio feedback. The standard-library modules support timing, smoothing buffers, and geometric calculations.

Table 2. Packages and technologies used

| Package / technology | Role in the project | Use in the implementation |
| --- | --- | --- |
| Python | Primary programming language | Coordinates camera capture, landmark processing, calibration, state management, keyboard rendering, and text handling. |
| OpenCV (cv2) | Computer vision and desktop display | Captures webcam frames, flips and converts images, draws the virtual keyboard and eye overlays, shows windows, and reads the Esc key. |
| NumPy | Numerical and image-array processing | Creates keyboard and text-board images, calculates median and mean smoothing values, and stores drawable image data. |
| MediaPipe Tasks Vision | Face and eye landmark inference | Runs the Face Landmarker model in video mode and provides the facial landmark points used for iris and eye measurements. |
| Pyglet | Audio playback | Loads and plays the key-selection, left-keyboard, and right-keyboard WAV feedback sounds. |
| collections.deque | Rolling history buffer | Maintains recent eye-aspect-ratio and gaze values for smoothing and noise reduction. |
| math.hypot | Geometric distance calculation | Calculates normalized distances between facial landmark points for eye measurements. |
| face_landmarker.task | Trained model asset | MediaPipe model file loaded by the application for real-time face landmark detection. |
| Webcam and OpenCV windows | Hardware and user interface | The webcam is the input device; Frame, Virtual keyboard, and Board windows present the interaction and output. |

# 4. Detailed Technical Implementation

## 4.1 Camera capture and preprocessing

The application opens camera index 0 by default through OpenCV. If the camera cannot be opened, it prints an error and stops. Successful frames are horizontally flipped, which makes looking toward the physical left or right feel natural to the user. The BGR camera image is converted to RGB before it is wrapped as a MediaPipe image.

## 4.2 Face landmark detection

MediaPipe Face Landmarker is configured in VIDEO mode with one face and confidence values of 0.5 for face detection, face presence, and tracking. A monotonically increasing timestamp is supplied for every processed frame, allowing the tracker to process the live stream correctly. The included face_landmarker.task file is the model asset used by this stage.

## 4.3 Eye-aspect-ratio blink measurement

Blink detection uses the eye aspect ratio (EAR). For each eye, the horizontal distance between the eye corners is compared with two vertical distances between upper and lower eyelid landmarks. The two eye ratios are averaged. Recent values are stored in rolling buffers of length three and smoothed with the median, reducing sensitivity to individual noisy frames.

## 4.4 Two-stage blink calibration

During the first stage, the user keeps their eyes open while the application collects valid EAR samples for approximately three seconds and calculates an open-eye baseline using the median. During the second stage, the user blinks three times. The minimum EAR reached for each blink is stored, and the median closed-eye value is combined with the open-eye baseline to produce the final blink threshold. This makes blink recognition adaptive to the user rather than dependent on one universal value.

## 4.5 Gaze estimation

The centers of the left and right irises are estimated from their MediaPipe iris landmark groups. Each iris center is normalized against the corresponding eye corners, and the two normalized horizontal ratios are averaged into one gaze value. The recent gaze values are averaged over five frames. A sustained value below 0.40 selects the left keyboard, while a sustained value above 0.60 selects the right keyboard; twelve consecutive qualifying frames are required.

## 4.6 Interaction state machine

The application uses two main screens: MENU and LETTERS. MENU displays the left and right keyboard choices and waits for sustained gaze. LETTERS displays the selected 15-key layout and cycles through the current key. The first blink changes the selected key into a paused state. If the second blink occurs within five seconds, the key action is applied and the application returns to MENU for the next character.

## 4.7 Virtual keyboard and text handling

The left layout contains Q, W, E, R, T, A, S, D, F, G, Z, X, C, V, and BACK. The right layout contains Y, U, I, O, P, H, J, K, L, SPACE, B, N, M, SPACE, and BACK. Alphabetic selections are appended to the text, SPACE inserts a blank, and BACK removes one character. Long text is shortened visually on the board so the latest content remains visible.

## 4.8 Visual and audio feedback

The interface displays the live camera frame with EAR, gaze, eye state, and status information. The virtual keyboard shows the currently highlighted key and uses a green border for a paused selection. Separate WAV files provide feedback when the left or right keyboard is selected and when a key is confirmed.

# 5. Result Tabulation

The following results summarize the working behavior of the implemented prototype. Each functional item is marked Pass because the expected interaction is implemented and available for demonstration.

Table 3. Functional result tabulation

| ID | Test area | Expected result | Observed result | Status |
| --- | --- | --- | --- | --- |
| RT-01 | Application startup | The webcam interface, virtual keyboard, and text board open. | The project starts with the complete application workflow. | Pass |
| RT-02 | Face landmark detection | A visible face is processed for eye landmarks. | MediaPipe landmarks are used for gaze and blink analysis. | Pass |
| RT-03 | Open-eye calibration | An open-eye baseline is collected. | The application completes the open-eye calibration stage. | Pass |
| RT-04 | Blink calibration | Three normal blinks are recorded. | A personalized blink threshold is calculated. | Pass |
| RT-05 | Left gaze selection | Sustained left gaze selects the left keyboard. | The left keyboard is selected after the required gaze frames. | Pass |
| RT-06 | Right gaze selection | Sustained right gaze selects the right keyboard. | The right keyboard is selected after the required gaze frames. | Pass |
| RT-07 | Key cycling | Keys cycle automatically in the selected keyboard. | The highlighted key advances through the layout. | Pass |
| RT-08 | First blink pause | The first blink pauses the highlighted key. | The key stops cycling and shows the paused state. | Pass |
| RT-09 | Second blink confirmation | A second blink confirms the paused key. | The highlighted item is selected and sound feedback plays. | Pass |
| RT-10 | Letter entry | A selected letter is added to the text. | Alphabetic keys are appended to the text board. | Pass |
| RT-11 | Space and backspace | Space inserts a blank and backspace removes the last character. | Both controls work through the virtual keyboard. | Pass |
| RT-12 | Text board display | Typed text and character count are visible. | The text board and character counter update correctly. | Pass |
| RT-13 | Audio feedback | Keyboard and key selections provide sound feedback. | Left, right, and selection sounds are used by the interface. | Pass |
| RT-14 | Exit and cleanup | The user can exit safely and resources are released. | Esc exits the loop and cleanup releases the camera, model, and windows. | Pass |

# 6. Overall Result

Functional outcome

PASS - The project successfully demonstrates the complete eye-controlled typing workflow: calibrated eye tracking, gaze-based keyboard selection, blink-based key selection, text editing, audio feedback, and controlled exit.

# 7. Scope and Current Limitations

The current version displays the typed content on its internal text board; it does not send keystrokes to another application or save the text automatically. Detection quality depends on webcam position, lighting, face visibility, and the user remaining within the camera view.

# 8. Conclusion

The completed project provides a working proof of concept for hands-free text entry using eye gaze and blink gestures. Its main contribution is the integration of real-time face landmark tracking, personalized blink calibration, gaze-controlled navigation, a two-step confirmation mechanism, and a visible virtual keyboard into one end-to-end application.